Day –9 : Vlan Trunking and Access port Theory

Step 1: Connect the Switches and Configure the Trunk Link

- Place Switch1 (e.g., 2960 or 3560) and Switch2 in your workspace.

- Connect FastEthernet 0/1 on Switch1 to FastEthernet 0/1 on Switch2 using a Copper Crossover cable (Note: Modern switches auto-detect straight-through cables via Auto-MDIX, but standard lab syntax calls for crossover between like devices).

- Access the CLI of Switch1 and configure the trunk:

Plaintext

Switch> enable

Switch# configure terminal

Switch(config)# hostname Switch1

Switch1(config)# interface FastEthernet 0/1

Switch1(config-if)# switchport trunk encapsulation dot1q

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# exit

(Note: 2960 series switches only support 802.1Q and will automatically skip the switchport trunk encapsulation dot1q command.)

- Repeat the CLI configuration on Switch2:

Plaintext

Switch> enable

Switch# configure terminal

Switch(config)# hostname Switch2

Switch2(config)# interface FastEthernet 0/1

Switch2(config-if)# switchport trunk encapsulation dot1q

Switch2(config-if)# switchport mode trunk

Switch2(config-if)# exit

Step 2: Create VLANs and Assign Access Ports on Switch1

- Create VLANs 10, 20, and 30 on Switch1:

Plaintext

Switch1(config)# vlan 10

Switch1(config-vlan)# name VLAN_10

Switch1(config-vlan)# vlan 20

Switch1(config-vlan)# name VLAN_20

Switch1(config-vlan)# vlan 30

Switch1(config-vlan)# name VLAN_30

Switch1(config-vlan)# exit

- Assign access ports on Switch1:

Plaintext

Switch1(config)# interface FastEthernet 0/2

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 10

Switch1(config)# interface FastEthernet 0/3

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 20

Switch1(config)# interface FastEthernet 0/4

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 30

Switch1(config-if)# exit

- Create the same VLANs (10, 20, 30) on Switch2:

Plaintext

Switch2(config)# vlan 10

Switch2(config)# vlan 20

Switch2(config)# vlan 30

Switch2(config)# exit

Step 3: Connect PCs and Verify VLAN Isolation

- Add PC0, PC1, and PC2 to the workspace.

- Connect them to Switch1:

  - PC0 → FastEthernet 0/2 (VLAN 10) — IP: 192.168.10.10 / 255.255.255.0

  - PC1 → FastEthernet 0/3 (VLAN 20) — IP: 192.168.10.20 / 255.255.255.0

  - PC2 → FastEthernet 0/4 (VLAN 30) — IP: 192.168.10.30 / 255.255.255.0

- Verification:

  - Ping from PC0 to PC1 or PC2: The ping fails. VLANs break up broadcast domains, so traffic cannot cross between different VLANs without a Layer 3 router or Layer 3 switch.

  - Connect another PC (PC3) to Switch2 on a port assigned to VLAN 10 (192.168.10.11). Ping from PC0 to PC3: The ping succeeds because traffic travels across the trunk link within the same VLAN.

Step 4: Changing Trunk Encapsulation to ISL

- To test ISL encapsulation, enter the commands on a Cisco Multilayer switch (e.g., 3560 in Packet Tracer), as standard 2960 switches do not support Cisco's legacy ISL protocol:

Plaintext

Switch1(config)# interface FastEthernet 0/1

Switch1(config-if)# switchport trunk encapsulation isl

Switch1(config-if)# switchport mode trunk

- Apply the matching configuration on Switch2:

Plaintext

Switch2(config)# interface FastEthernet 0/1

Switch2(config-if)# switchport trunk encapsulation isl

Switch2(config-if)# switchport mode trunk

- Result:

  - If both ends are set to ISL, traffic passes normally.

  - If one end is ISL and the other is 802.1Q, the link drops and VLAN traffic will fail to pass because the encapsulation methods are mismatched.

Step 5: Accidental Trunk Port Configuration on Switch2 (Simple Explanation)

Configuration:

On Switch2, set FastEthernet 0/5 as a trunk port instead of an access port for VLAN 20:

Plaintext

Switch2(config)# interface FastEthernet 0/5

Switch2(config-if)# switchport mode trunk

Then plug a PC (PC-Test) into FastEthernet 0/5 with an IP address in the VLAN 20 subnet.

What happens and why? (In Easy Language)

- The PC will lose regular network connectivity.

  - Why? A PC's network card expects normal, untagged data frames. A trunk port expects to talk to another switch and sends tagged data packets (carrying a VLAN ID tag). Because the PC does not understand VLAN tags, it drops the incoming data packets.

- Native VLAN Leakage:

  - By default, a trunk port leaves VLAN 1 traffic untagged (Native VLAN).

  - Therefore, the PC on port 0/5 will accidentally receive traffic intended for VLAN 1 instead of VLAN 20.

- Summary of Impact:

  - The PC cannot talk to other computers in VLAN 20.

  - The PC loses access to its intended network and causes a security risk by potentially listening to VLAN 1 network management traffic.