Day -1 : Static Routing Theory

1. Serial Connection & IP Addressing Setup :

- Add Routers: Drag two 2911 (or 1841) routers onto the canvas named Router1 and Router2.

- Add Serial Modules:

  - Double-click Router1 → Physical tab → Power OFF the router.

  - Drag the HWIC-2T module into an open slot → Power ON the router.

  - Repeat this for Router2.

- Connect Serial Cable: Select Serial DCE cable → Click Router1 (Serial0/3/0) → Click Router2 (Serial0/3/0).

Interface Configuration Commands

On Router1:

Plaintext

Router> enable

Router# configure terminal

Router1(config)# interface Serial0/3/0

Router1(config-if)# ip address 10.0.0.1 255.255.255.252

Router1(config-if)# clock rate 64000

Router1(config-if)# no shutdown

Router1(config-if)# exit

On Router2:

Plaintext

Router> enable

Router# configure terminal

Router2(config)# interface Serial0/3/0

Router2(config-if)# ip address 10.0.0.2 255.255.255.252

Router2(config-if)# no shutdown

Router2(config-if)# exit

2. Static Route Configuration & Connectivity Test :

Assuming Router2 has a LAN interface GigabitEthernet0/0 with IP 192.168.2.1/24 and a connected PC with IP 192.168.2.10 (Default Gateway: 192.168.2.1).

Configure Static Route on Router1

Plaintext

Router1(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2

Verification Step

To verify connectivity from Router1 to the PC:

Plaintext

Router1# ping 192.168.2.10

Verification: Success rate should be 80% to 100% (5/5 or 4/5 replies received).

3. Zomato-Style Delivery Network Simulation (Ahmedabad to Surat)

Network Topology & IP Scheme

- Ahmedabad LAN (Router1 Gi0/0): 192.168.10.0/24 (Delivery App Server: 192.168.10.10)

- WAN Link (Serial Link): 10.0.0.0/30

- Surat LAN (Router2 Gi0/0): 192.168.20.0/24 (Delivery Partner Device: 192.168.20.10)

Router Configurations

On Ahmedabad Router (Router1):

Plaintext

Ahmedabad(config)# interface GigabitEthernet0/0

Ahmedabad(config-if)# ip address 192.168.10.1 255.255.255.0

Ahmedabad(config-if)# no shutdown

Ahmedabad(config-if)# exit

! Static Route to reach Surat LAN via Serial Link

Ahmedabad(config)# ip route 192.168.20.0 255.255.255.0 10.0.0.2

On Surat Router (Router2):

Plaintext

Surat(config)# interface GigabitEthernet0/0

Surat(config-if)# ip address 192.168.20.1 255.255.255.0

Surat(config-if)# no shutdown

Surat(config-if)# exit

! Static Route to reach Ahmedabad LAN via Serial Link

Surat(config)# ip route 192.168.10.0 255.255.255.0 10.0.0.1

Verification Step

Open Command Prompt on Ahmedabad Server (192.168.10.10) and ping Surat Partner (192.168.20.10):

DOS

C:\> ping 192.168.20.10

Verification: You will receive Reply from 192.168.20.10: bytes=32 time<1ms TTL=126, confirming two-way city communication.

4. Advantages & Disadvantages of Static Routing :

| Category | Advantage / Disadvantage | Real-Life Application Example (Swiggy / IRCTC) |
| --- | --- | --- |
| Advantage 1 | Predictable Performance & Security: Manual routes don't change unexpectedly and take up zero router CPU/RAM power. | Swiggy Payment Gateway: High-security payment processing servers use fixed, manual routes to ensure payment requests always go through a specific secured firewall without risk of route high-jacking. |
| Advantage 2 | No Extra Bandwidth Waste: Routers don't exchange constant network updates over the link. | IRCTC Ticket Printing Kiosks: Local station printing kiosks send requests over a dedicated static route without cluttering the primary network with extra routing traffic. |
| Disadvantage 1 | No Automatic Failover: If a cable or link snaps, static routes fail completely until an engineer fixes them manually. | Swiggy Live Order Tracking: If a fiber link between nodes cuts, static routing won't automatically switch to a backup line, causing live delivery tracking updates to freeze for users. |
| Disadvantage 2 | Hard to Scale in Large Networks: Every new node requires manually updating static routes on every single connected router. | IRCTC Tatkal Booking Load: Adding hundreds of temporary booking servers across India for peak hours would be painful with static routing, as network admins would have to update thousands of router tables manually. |