Day -10 : Inter-Vlan Routing

1.Create VLANs and Assign Switch Ports:

- Drag a 2960 Switch and 3 PCs onto the workspace:

  - PC-Foodies (Zomato)

  - PC-Shoppers (Flipkart)

  - PC-Streamers (Spotify)

- Open the Switch CLI and configure the VLANs:

Plaintext

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name Foodies

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name Shoppers

Switch(config-vlan)# exit

Switch(config)# vlan 30

Switch(config-vlan)# name Streamers

Switch(config-vlan)# exit

- Assign Switchport access ports to each PC:

Plaintext

Switch(config)# interface FastEthernet0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

Switch(config)# interface FastEthernet0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# exit

Switch(config)# interface FastEthernet0/3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 30

Switch(config-if)# exit

2.Configure Trunk Link and Router Sub-Interfaces:

- Set the Switch link connected to the Router (e.g., GigabitEthernet0/1) to Trunk Mode:

Plaintext

Switch(config)# interface GigabitEthernet0/1

Switch(config-if)# switchport mode trunk

Switch(config-if)# exit

- Configure the Router (1941 or 2911) sub-interfaces with 802.1Q encapsulation:

Plaintext

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface GigabitEthernet0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config-subif)# exit

Router(config)# interface GigabitEthernet0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config-subif)# exit

Router(config)# interface GigabitEthernet0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Router(config-subif)# exit

3.Assign IP Addresses and Default Gateways to PCs:

Configure the Desktop > IP Configuration tab on each PC with the following settings:

| Device | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC-Foodies | VLAN 10 (Foodies) | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC-Shoppers | VLAN 20 (Shoppers) | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| PC-Streamers | VLAN 30 (Streamers) | 192.168.30.10 | 255.255.255.0 | 192.168.30.1 |

4.Simulate, Identify, and Fix Inter-VLAN Ping Failure:

Simulated Scenario:

A misconfiguration prevents PC-Shoppers from pinging PC-Streamers (Request Timed Out).

Common Root Causes & Diagnostics:

- Incorrect Encapsulation Tag on Sub-interface: Sub-interface Gi0/0.30 was misconfigured with encapsulation dot1Q 20 instead of 30.

- Incorrect Switchport VLAN: FastEthernet0/3 on the Switch was assigned to VLAN 10 instead of VLAN 30.

- Missing/Wrong Default Gateway: PC-Streamers has its gateway set to 192.168.20.1 instead of 192.168.30.1.

Fixing the Sub-Interface Encapsulation Issue (Most Common):

Plaintext

Router(config)# interface GigabitEthernet0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.30.1 255.255.255.0

Verification:

Open the Command Prompt on PC-Shoppers (192.168.20.10) and run:

Bash

ping 192.168.30.10

Result: Reply from 192.168.30.10: bytes=32 time<1ms TTL=127 (Connectivity Restored).

Router-on-a-Stick vs. Layer 3 Switch Routing

| Routing Method | Advantages | Disadvantages |
| --- | --- | --- |
| Router-on-a-Stick | 1. Cheaper: Works with lower-cost Layer 2 switches and a standard router. 2. Advanced Features: Supports deep packet inspection, advanced NAT, and complex firewall rules easily. | 1. Traffic Bottleneck: All traffic between VLANs travels over a single physical cable. 2. Slower Speed: Traffic is processed in software/CPU rather than hardware ASIC chips. |
| Layer 3 Switch | 1. Extremely Fast: Routes traffic directly in hardware (ASICs) at wire speed. 2. No Single Cable Bottleneck: Internal backplane bandwidth carries traffic without clogging physical router links. | 1. More Expensive: Layer 3 capable switches cost significantly more. 2. Limited Router Features: Lacks advanced WAN features like NAT, VPN tunneling, or deep security filtering. |