Day -11 : Practice Practical

Step 1: Network Topology Setup

- Add the Devices:

  - Switches: Place 3x 2960 Switches (Switch1, Switch2, Switch3).

  - PCs: Place 6x PCs (2 connected to each switch).

  - Router: Place 1x 2811 or 1941 Router (for Router-on-a-Stick inter-VLAN routing).

- Cable the Connections:

  - PC to Switch: Use Copper Straight-Through cables.

  - Switch to Switch: Use Copper Cross-Over (or Straight-Through, as Auto-MDIX will auto-detect).

  - Switch1 to Router: Connect Switch1 FastEthernet0/1 to Router GigabitEthernet0/0 (or FastEthernet0/0).

Step 2: Configure VLANs on All Switches

You need to create VLAN 10 and VLAN 20 on all three switches and assign the PC access ports.

Configuration on Switch 1, 2, and 3:

Click on each switch, go to the CLI tab, and enter:

Plaintext

enable

configure terminal

! Create VLANs

vlan 10

name Sales

vlan 20

name Support

exit

Assign PC Ports (Access Mode):

- PC 1 (Sales): Connect to Fa0/2 → Assign to VLAN 10

- PC 2 (Support): Connect to Fa0/3 → Assign to VLAN 20

Run these commands on the switch port where the PC is connected:

Plaintext

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

exit

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

exit

(Repeat port assignments for Switch 2 and Switch 3).

Step 3: Configure Trunk Ports Between Switches

Trunk lines allow traffic from both VLAN 10 and VLAN 20 to travel between switches.

On Switch 1 (Interconnecting ports, e.g., Fa0/10 to Switch 2, Fa0/11 to Switch 3):

Plaintext

configure terminal

interface range FastEthernet 0/10 - 11

switchport mode trunk

exit

On Switch 2 & Switch 3 (Interconnecting port connected back to Switch 1):

Plaintext

configure terminal

interface FastEthernet 0/10

switchport mode trunk

exit

Verify Trunking:

Run this command on any switch in Privileged EXEC mode (# prompt):

Plaintext

show interfaces trunk

- For your assignment screenshot submission: Click on the Switch CLI window after running show interfaces trunk and take a screenshot showing active trunking status and allowed VLANs (1,10,20).

Step 4: Configure Inter-VLAN Routing (Router-on-a-Stick)

Connect Switch1's Fa0/1 port to the Router's Gi0/0 port.

- Set the Switch Port to Trunk:

Plaintext

Switch1(config)# interface FastEthernet 0/1

Switch1(config-if)# switchport mode trunk

- Configure Router Sub-Interfaces:

Click on the Router → CLI tab:

Plaintext

enable

configure terminal

! Turn on the main interface

interface GigabitEthernet 0/0

no shutdown

exit

! Sub-interface for VLAN 10 (Sales)

interface GigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

! Sub-interface for VLAN 20 (Support)

interface GigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

Step 5: Configure PC IP Addresses & Default Gateways

Click on each PC → Desktop tab → IP Configuration:

| Device / Department | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC in VLAN 10 (Sales) | 192.168.10.X (e.g., 192.168.10.10) | 255.255.255.0 | 192.168.10.1 |
| PC in VLAN 20 (Support) | 192.168.20.X (e.g., 192.168.20.10) | 255.255.255.0 | 192.168.20.1 |

Important: The Default Gateway on the PC must match the sub-interface IP address configured on the router for that VLAN.

Step 6: Testing & Troubleshooting Ping

- Open Command Prompt on a VLAN 10 PC (192.168.10.10).

- Ping a VLAN 20 PC (192.168.20.10):

Plaintext

ping 192.168.20.10

- Note: The first 1 or 2 pings might time out due to ARP resolution. If it shows Reply from 192.168.20.10, inter-VLAN routing is working successfully.

Troubleshooting Checklist (If Ping Fails):

- Default Gateway: Check if the PC has 192.168.10.1 (or 192.168.20.1) entered as its Default Gateway.

- Encapsulation: Verify encapsulation dot1Q 10 and encapsulation dot1Q 20 were typed under the correct router sub-interfaces.

- Trunk Port to Router: Ensure the switch port connecting to the router (Fa0/1) is set to switchport mode trunk