Day -3 : Configure RIP Protocol

Step 1: Set Up the Network Topology & Assign IP Addresses

We will create a linear network of three routers (R1 — R2 — R3) using GigabitEthernet or FastEthernet links so you don't need extra serial WAN interface cards.

IP Address Plan

- Network 1 (LAN 1): 192.168.10.0/24 (Left side)

- Network 2 (R1-R2 Link): 10.1.1.0/24 (Middle left)

- Network 3 (R2-R3 Link): 10.2.2.0/24 (Middle right)

- Network 4 (LAN 2): 192.168.20.0/24 (Right side)

Router 1 Configuration (R1)

Click on R1, go to the CLI tab, and enter the following commands:

Plaintext

enable

configure terminal

hostname R1

interface GigabitEthernet0/0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 10.1.1.1 255.255.255.0

no shutdown

exit

Verification: Run show ip interface brief in privileged EXEC mode (R1#) to verify both interfaces display up/up.

Router 2 Configuration (R2)

Click on R2, go to the CLI tab, and enter:

Plaintext

enable

configure terminal

hostname R2

interface GigabitEthernet0/0/0

ip address 10.1.1.2 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 10.2.2.1 255.255.255.0

no shutdown

exit

Verification: Run show ip interface brief on R2 to confirm both links are active (up/up).

Router 3 Configuration (R3)

Click on R3, go to the CLI tab, and enter:

Plaintext

enable

configure terminal

hostname R3

interface GigabitEthernet0/0/0

ip address 10.2.2.2 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

Verification: Run show ip interface brief on R3 to ensure both interfaces show up/up.

Step 2: Enable RIP Version 2 & Verify Routes

Enable RIPv2 on R1

Plaintext

R1# configure terminal

R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# no auto-summary

R1(config-router)# network 192.168.10.0

R1(config-router)# network 10.0.0.0

R1(config-router)# exit

Enable RIPv2 on R2

Plaintext

R2# configure terminal

R2(config)# router rip

R2(config-router)# version 2

R2(config-router)# no auto-summary

R2(config-router)# network 10.0.0.0

R2(config-router)# exit

Enable RIPv2 on R3

Plaintext

R3# configure terminal

R3(config)# router rip

R3(config-router)# version 2

R3(config-router)# no auto-summary

R3(config-router)# network 10.0.0.0

R3(config-router)# network 192.168.20.0

R3(config-router)# exit

Verification

Wait about 30 seconds for RIP to send update messages. Then check the routing table on R1:

Plaintext

R1# show ip route

What you should see: Look for lines starting with R (which stands for RIP):

- R 10.2.2.0/24 [120/1] via 10.1.1.2

- R 192.168.20.0/24 [120/2] via 10.1.1.2

Understanding [120/2]: 120 is the Administrative Distance of RIP, and 2 is the hop count (the network is 2 router jumps away).

Step 3: Observe Hop Count Metric Changes

To see how RIP calculates distance using hops:

- Insert a new router (R4) in the middle between R2 and R3.

- Remove the direct link between R2 and R3.

- Connect R2 to R4, and R4 to R3.

- Assign IP addresses for the new subnet between R4 and R3 (e.g., 10.3.3.0/24), enable RIPv2 on R4, and advertise network 10.0.0.0.

Observing the Change

Now go back to R1 and check the routing table again:

Plaintext

R1# show ip route

Observation: The route to 192.168.20.0/24 on R1 will now show: R 192.168.20.0/24 [120/3] via 10.1.1.2

Conclusion: Adding R4 added an extra physical router jump. The hop count increased from 2 to 3. RIP always selects routes based purely on the lowest number of router jumps (hops).

Step 4: Analyze RIP Update Packets in Simulation Mode

- Switch Packet Tracer from Realtime Mode to Simulation Mode (bottom right corner, or press Shift + S).

- Click Show All/None under Event List Filters to uncheck everything, then click Edit Filters.

- Under the IPv4 tab, check RIP.

- Click Play / Capture Forward button.

- Watch the envelope icons move across the links between routers.

Analyzing the Packet Contents

- Click on any envelope traveling between R1 and R2.

- Look at the In Layer / Out Layer details:

  - Source IP: The interface IP sending the update (e.g., 10.1.1.1).

  - Destination IP: 224.0.0.9 (RIPv2's multicast address for routing updates).