Day -4 : Configure OSPF Protocol

Step 1: Set Up the Topology

- Drag 3 Routers (e.g., 2911 or 1941) onto the workspace: R1, R2, and R3.

- Connect them in a triangle using Copper Cross-Over (or Copper Straight-Through if using GigabitEthernet ports):

  - R1 G0/0 → R2 G0/0 (Network: 10.1.1.0/24)

  - R2 G0/1 → R3 G0/0 (Network: 10.2.2.0/24)

  - R3 G0/1 → R1 G0/1 (Network: 10.3.3.0/24)

Step 2: Assign IP Addresses & Test Connectivity

Click each router, open the CLI tab, press Enter, and enter these commands to configure the IP addresses and activate the interfaces.

Router R1

Plaintext

enable

configure terminal

interface GigabitEthernet0/0

ip address 10.1.1.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 10.3.3.1 255.255.255.0

no shutdown

exit

Router R2

Plaintext

enable

configure terminal

interface GigabitEthernet0/0

ip address 10.1.1.2 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 10.2.2.1 255.255.255.0

no shutdown

exit

Router R3

Plaintext

enable

configure terminal

interface GigabitEthernet0/0

ip address 10.2.2.2 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 10.3.3.2 255.255.255.0

no shutdown

exit

Verify Base Connectivity:

On R1, test the direct links by running:

Plaintext

ping 10.1.1.2

ping 10.3.3.2

Both pings should show Success rate is 80 or 100 percent.

Step 3: Configure OSPF

Enable OSPF (Process ID 1) in Area 0 on all routers to advertise their directly connected networks.

Router R1

Plaintext

configure terminal

router ospf 1

network 10.1.1.0 0.0.255.255 area 0

network 10.3.3.0 0.0.255.255 area 0

exit

Router R2

Plaintext

configure terminal

router ospf 1

network 10.1.1.0 0.0.255.255 area 0

network 10.2.2.0 0.0.255.255 area 0

exit

Router R3

Plaintext

configure terminal

router ospf 1

network 10.2.2.0 0.0.255.255 area 0

network 10.3.3.0 0.0.255.255 area 0

exit

Step 4: Verify OSPF Adjacency

After a few seconds, you will see OSPF log messages on the CLI saying %OSPF-5-ADJCHG: Process 1, Nbr ... from LOADING to FULL.

To check neighbor relationships, exit to privileged mode and run:

Plaintext

show ip ospf neighbor

Expected Result:

You will see your neighbor routers listed. Look under the State column:

- It should display FULL/DR, FULL/BDR, or FULL/DROTHER.

- The FULL state confirms that the routers have fully synchronized their database and formed an adjacency.

To view learned OSPF routes on R1, run:

Plaintext

show ip route ospf

You will see a route to 10.2.2.0/24 with an OSPF cost (metric).

Step 5: Change OSPF Cost & Observe Route Changes

By default, OSPF chooses the path with the lowest metric (cost). We will manually increase the cost on R1's direct link to R2 to force R1 to reroute traffic to R2 through R3 instead.

- Check current route to 10.2.2.0 on R1:

Plaintext

show ip route ospf

Notice it routes traffic directly through R2 (10.1.1.2) with a low cost.

- Increase the interface cost on R1's interface toward R2:

Plaintext

configure terminal

interface GigabitEthernet0/0

ip ospf cost 500

exit

- Verify the routing table update on R1:

Plaintext

show ip route ospf