Day -5 : Configure OSPF Multiple Area

1. Adding Serial Modules to Routers

Cisco 2911 routers do not have built-in serial ports by default, so you must install them manually:

- Click on R1 > go to the Physical tab.

- Click the Power Switch to turn off the router.

- Under the Modules menu on the left, select HWIC-2T.

- Drag and drop the HWIC-2T module into an open slot on the right side of the router.

- Turn the power switch back ON.

- Repeat these exact steps for R2 and R3.

2. Physical Connections

Use the Serial DCE Cable (the red lightning cable with a clock icon in the Connections palette):

- Connect R1 Serial0/0/0 to R2 Serial0/0/0.

- Connect R2 Serial0/0/1 to R3 Serial0/0/0.

3. Router CLI Configuration Commands

Replace the previous GigabitEthernet interface commands with Serial commands:

R1 (Area 1 Internal Router)

Plaintext

enable

configure terminal

hostname R1

interface GigabitEthernet0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface Serial0/0/0

ip address 10.1.1.1 255.255.255.252

no shutdown

exit

router ospf 1

router-id 1.1.1.1

network 192.168.10.0 0.0.0.255 area 1

network 10.1.1.0 0.0.0.3 area 1

R2 (ABR - Area Border Router)

Plaintext

enable

configure terminal

hostname R2

interface Serial0/0/0

ip address 10.1.1.2 255.255.255.252

clock rate 64000

no shutdown

exit

interface Serial0/0/1

ip address 10.2.2.1 255.255.255.252

clock rate 64000

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 10.1.1.0 0.0.0.3 area 1

network 10.2.2.0 0.0.0.3 area 0

(Note: clock rate 64000 is applied on the DCE end of serial links to generate clock signal timing).

R3 (Area 0 Backbone Router)

Plaintext

enable

configure terminal

hostname R3

interface Serial0/0/0

ip address 10.2.2.2 255.255.255.252

no shutdown

exit

interface GigabitEthernet0/1

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 3.3.3.3

network 10.2.2.0 0.0.0.3 area 0

network 192.168.20.0 0.0.0.255 area 0

4. Simulating Failure with Serial Interface

To run Step 4 using Serial links:

- Shut down interface Serial0/0/0 on R2:

Plaintext

R2(config)# interface Serial0/0/0

R2(config-if)# shutdown

- Verify route removal on R3: show ip route

- Bring the interface back up:

Plaintext

R2(config-if)# no shutdown

- Observe recovery time (~10 to 40 seconds) until show ip ospf neighbor shows FULL status and pings between PC0 and PC1 succeed.