Day -6 : Wireless Standard and Configuration

Step 1: Connect to Your Router and Find Default Settings

- Connect your device: Turn on your Wi-Fi router. On your phone or laptop, look for the network name printed on the sticker on the back or bottom of the router. Connect using the default password listed on that sticker.

- Access the router settings: Open any web browser (Chrome, Safari, etc.) and type your router's default IP address into the address bar.

  - Common addresses: 192.168.1.1 or 192.168.0.1

- Log in: Enter the admin username and password (also found on the router's sticker).

- Note down default details: Go to the Wireless or Wi-Fi Settings tab and write down:

  - SSID: Your Wi-Fi network's name.

  - Channel: The current channel (it usually defaults to Auto).

Step 2: Change the Wi-Fi Channel to Reduce Interference

- Change the channel manually:

  - In your router’s Wireless Settings, locate the Channel option.

  - Change it from Auto to a fixed channel—1, 6, or 11 are recommended for 2.4 GHz networks because they do not overlap with each other.

  - Click Save or Apply.

- Check for interference:

  - Download a free Wi-Fi analyzer app on your phone (such as Wi-Fi Analyzer on Android or AirPort Utility on iOS).

  - Open the app and look at the graph showing nearby networks.

  - Verify that your chosen channel has fewer overlapping neighbor networks compared to other channels.

Step 3: Enable WPA2/WPA3 Security and Set a Strong Password

- Navigate to Security Settings: Go to Wireless > Wireless Security inside your router settings.

- Select Security Type: Choose WPA2-PSK (AES) or WPA3 (if supported by your router). Avoid outdated options like WEP or WPA.

- Set a Password: Create a strong password (at least 8–12 characters mixing uppercase letters, numbers, and special characters).

- Save & Test:

  - Click Save. Your router may restart its Wi-Fi signal, disconnecting your devices.

  - On your phone or laptop, forget the old Wi-Fi network connection and reconnect using your new password.

Step 4: Configure MAC Address Filtering

- Find your main device's MAC Address:

  - Android / iOS: Go to Settings > About Phone (or Wi-Fi Settings) > look for Wi-Fi MAC Address.

  - Windows: Open Command Prompt, type getmac, and press Enter.

  - Mac: Go to System Settings > Network > Wi-Fi > Advanced to view the MAC address.

- Enable MAC Filtering on the router:

  - Go to Security or Advanced > MAC Filtering (or Access Control).

  - Turn MAC Filtering ON and set the rule to Allow (Whitelist mode).

  - Add your main device's MAC address to the allowed list and save.

- Test with another device:

  - Try connecting a second device (whose MAC address was not added to the list) to your Wi-Fi network.

  - Result: The second device will fail to connect or won't be assigned an IP address, proving that access control is working.

Step 5: Create an Access Control List (ACL) to Block a Website

- Find Access Control / Parental Controls:

  - Go to Security, Access Control, or Parental Controls in your router menu.

- Create a Blocking Rule:

  - Add a new rule or domain filter.

  - Select URL Blocking or Domain Blocking.

  - Enter the domain name you want to block: [www.youtube.com](https://www.youtube.com) (or youtube.com).

  - Apply the rule to all devices or your specific Wi-Fi network.

- Save and Apply: Click Save and restart your browser on your connected device.

- Test the Block:

  - Open a web browser on your phone or laptop connected to the Wi-Fi and attempt to visit [www.youtube.com](https://www.youtube.com).

  - Result: The page should fail to load, display a connection error, or show a site blocked notice.