Day -7 : Switching Basic  Theory

1. Initial State (2 PCs + 1 Switch)

When you first power on the switch and connect PC1 and PC2, no network traffic has crossed the switch yet. Switches do not know which device is attached to which port until traffic flows.

Running the command in the switch CLI:

Plaintext

Switch# show mac address-table

Observation:

The table is completely empty (or contains only dynamic internal switch entries). No end-device MAC addresses appear because the switch learns MAC addresses strictly from incoming frame headers, and no device has sent a frame yet.

2. Ping from PC1 to PC2

When you run ping <PC2-IP> from PC1:

- Step 1: PC1 sends an ARP request / ICMP echo packet wrapped in an Ethernet frame.

- Step 2: The frame arrives at the switch port (e.g., FastEthernet0/1). The switch inspects the Source MAC address (PC1) and saves it: Fa0/1 -> PC1's MAC.

- Step 3: Since PC2's MAC is not yet in the table, the switch floods the frame to all other ports.

- Step 4: PC2 receives the frame and sends a reply back.

- Step 5: The reply enters FastEthernet0/2. The switch inspects the Source MAC address (PC2) and saves it: Fa0/2 -> PC2's MAC.

Running the command on the switch:

Plaintext

Switch# show mac address-table

Mac Address Table

-------------------------------------------

VLAN    Mac Address       Type        Ports

----    -----------       --------    -----

1    0060.2F3A.1111    DYNAMIC     Fa0/1

1    00D0.D38E.2222    DYNAMIC     Fa0/2

Observation:

Both MAC addresses now appear in the table associated with their respective interface ports.

3. Adding PC3 and Pinging PC1

Connect PC3 to FastEthernet0/3. Before sending traffic, the switch still has no entry for PC3.

When PC3 pings PC1:

- PC3 generates a frame with its MAC address as the Source MAC and PC1's MAC address as the Destination MAC.

- As soon as the frame enters port Fa0/3, the switch reads the source address and immediately logs PC3's MAC address to Fa0/3.

- Next, the switch looks at the destination address (PC1). Because PC1 is already known on Fa0/1, the switch forwards the frame directly out of Fa0/1 without flooding it to PC2.

Running the command on the switch:

Plaintext

Switch# show mac address-table

Mac Address Table

-------------------------------------------

VLAN    Mac Address       Type        Ports

----    -----------       --------    -----

1    0060.2F3A.1111    DYNAMIC     Fa0/1

1    00D0.D38E.2222    DYNAMIC     Fa0/2

1    0060.4C99.3333    DYNAMIC     Fa0/3

Observation:

A third entry is added for Fa0/3 containing PC3's MAC address.

4. Frame Forwarding Step-by-Step Scenario

Given table:

- Port 1: 00:11:22:33:44:55

- Port 2: 66:77:88:99:AA:BB

Scenario: A frame enters Port 1 with Destination MAC: 66:77:88:99:AA:BB.

Step-by-Step Processing:

- Source Inspection: The switch receives the frame on Port 1 and checks the Source MAC address. If Port 1's source MAC isn't recorded or updated, it refreshes or learns it.

- Destination Lookup: The switch reads the Destination MAC address (66:77:88:99:AA:BB) and searches its MAC address table for a match.

- Match Found: The table shows that MAC 66:77:88:99:AA:BB is connected to Port 2.

- Unicast Forwarding: The switch forwards the frame only out of Port 2. It does not send the frame to any other ports, conserving bandwidth and ensuring privacy.

5. Concept Diagram & Explanation

Text Diagram

Plaintext

[ PC A ]                     [ PC B ]

(00:00:00:00:00:0A)          (00:00:00:00:00:0B)

|                            |

Port 1                       Port 2

+---------------------------------------+

|                 SWITCH                |

|                                       |

|         MAC Address Table             |

|   Port 1  <-->  00:00:00:00:00:0A     |

|   Port 2  <-->  00:00:00:00:00:0B     |

|   Port 3  <-->  00:00:00:00:00:0C     |

|   Port 4  <-->  00:00:00:00:00:0D     |

+---------------------------------------+

Port 3                       Port 4

|                            |

[ PC C ]                     [ PC D ]

(00:00:00:00:00:0C)          (00:00:00:00:00:0D)

Simple Explanation

A switch works like a post office directory for a local network. Every time a device sends data, the switch looks at who sent it (the source MAC) and writes down which port that sender is plugged into. When data needs to be delivered, the switch checks its list for the receiver's address; if it finds a match, it sends the data straight to that specific port. If the receiver's address isn't on the list yet, the switch broadcasts the data to everyone until the intended receiver replies and reveals its location.