Day -8 : Switch Vlan Configuration

1. Cisco Switch CLI Commands

Below are the exact commands needed to create the VLANs and assign the specified ports on a Cisco switch using CLI in Cisco Packet Tracer.

! Step 1: Enter Global Configuration Mode

Switch> enable

Switch# configure terminal

! Step 2: Create VLAN 10 and name it 'Engineering'

Switch(config)# vlan 10

Switch(config-vlan)# name Engineering

Switch(config-vlan)# exit

! Step 3: Create VLAN 20 and name it 'Marketing'

Switch(config)# vlan 20

Switch(config-vlan)# name Marketing

Switch(config-vlan)# exit

! Step 4: Assign ports FastEthernet0/1 and FastEthernet0/2 to VLAN 10

Switch(config)# interface range fastEthernet 0/1 - 2

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Switch(config-if-range)# exit

! Step 5: Assign ports FastEthernet0/3 and FastEthernet0/4 to VLAN 20

Switch(config)# interface range fastEthernet 0/3 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Switch(config-if-range)# exit

! Step 6: Save configuration

Switch(config)# end

Switch# copy running-config startup-config

2. VLAN Configuration Summary

| VLAN ID | VLAN Name | Assigned Ports |
| --- | --- | --- |
| 10 | Engineering | FastEthernet0/1, FastEthernet0/2 |
| 20 | Marketing | FastEthernet0/3, FastEthernet0/4 |

3. Benefits of Using VLANs in a College Campus Network

In a college environment with separate labs for Coding and Networking, VLANs provide crucial network separation and control.

WITHOUT VLANs (One Big Room)

+----------------------------------------+

|  [Coding PC] <---> [Networking PC]     |  Broadcasts & traffic

|      ^                    ^            |  are visible to everyone!

|      |                    |            |  (Higher risk, more noise)

|  [Coding PC] <---> [Networking PC]     |

+----------------------------------------+

WITH VLANs (Virtual Separate Rooms)

+--------------------+  +--------------------+

| VLAN 10 (Coding)   |  | VLAN 20 (Network)  |

| [PC 1] <-> [PC 2]  |  | [PC 3] <-> [PC 4]  |

|                    |  |                    |  Traffic is isolated!

| Isolated Traffic   |  | Isolated Traffic   |  (Safer & faster)

+--------------------+  +--------------------+

Benefit 1: Better Security and Isolation

- What it means: VLANs act like invisible walls between different groups of computers connected to the same switch.

- Why it helps the college: Networking students often experiment with network tools, custom protocols, or testing traffic that could disrupt other computers. By putting the Networking Lab on VLAN 20 and the Coding Lab on VLAN 10, any experimental traffic or security incidents in the networking lab are isolated and cannot interfere with or expose the coding lab computers.

Benefit 2: Reduced Network Traffic and Better Performance

- What it means: Computers on a network regularly send out "broadcast messages" (like a loudspeaker announcement meant for everyone). Without VLANs, every computer receives every broadcast message.