Day -5 : Troubleshooting Network Service

1. Local DNS Resolution Failure Simulation

Simulation Steps

- Open your text editor as an Administrator (Windows) or using sudo (macOS/Linux).

  - Windows: Open Notepad as Administrator →File →Open →C:\Windows\System32\drivers\etc\hosts

  - macOS/Linux: Open Terminal and run: sudo nano /etc/hosts

- Add the following line at the bottom of the file to point Spotify to an unroutable IP address:

Plaintext

127.0.0.2 www.spotify.com

- Save the file and flush your system's DNS cache:

  - Windows: ipconfig /flushdns

  - macOS: sudo dscacheutil -flushcache; sudo killall -HUP mDNSResponder

  - Linux: sudo systemd-resolve --flush-caches

Testing & Verification

Open your command line and ping the domain:

Bash

ping www.spotify.com

Documented Error Message

Depending on your Operating System, you will receive one of the following outputs:

- Windows:

Plaintext

Pinging www.spotify.com [127.0.0.2] with 32 bytes of data:

Reply from 127.0.0.2: bytes=32 time<1ms TTL=128

(Note: If the IP used is invalid or down, such as 192.0.2.1, Windows displays: Request timed out. or Destination host unreachable.)

- Linux / macOS:

Plaintext

PING www.spotify.com (127.0.0.2): 56 data bytes

Request timeout for icmp_seq 0

Verification: To verify the failure was triggered by local resolution, run nslookup [www.spotify.com](https://www.spotify.com). The DNS server will return Spotify's actual public IP address, proving that the operating system's local hosts file override is intercepting traffic before it reaches the network DNS server.

2. DHCP Misconfiguration (Wrong Gateway)

Scenario Setup

In Cisco Packet Tracer or GNS3:

- Configure a DHCP server pool with an incorrect gateway (e.g., Subnet: 192.168.1.0/24, Wrong Gateway: 192.168.1.250 instead of 192.168.1.1).

- Connect a PC client and switch its IP configuration mode from Static to DHCP.

Client Verification

- Packet Tracer PC GUI: Navigate to Desktop →IP Configuration. Observe that Default Gateway shows 192.168.1.250.

- Command Line Check:

DOS

ipconfig /all

Check the Default Gateway entry under the active Network Adapter.

Identification & Fix Process

1

Identify the Failure Point

Diagnostic Step

1.Identify the Failure Point:Diagnostic Step.

Run a traceroute to an external address (e.g., tracert 8.8.8.8). Notice that traffic drops immediately at the first hop (192.168.1.250), showing * * * Request timed out.

2

Verify Gateway Reachability

Diagnostic Step

2.Verify Gateway Reachability:Diagnostic Step.

Ping the gateway (ping 192.168.1.250). If there is no reply, or if an ARP lookup fails, the gateway address does not exist on the local link.

3

Correct Router/DHCP Server Config

Resolution Step

3.Correct Router/DHCP Server Config:Resolution Step.

Log into the DHCP Server (or Cisco Router) CLI and update the default router IP for the DHCP pool:

Plaintext

ip dhcp pool LAN_POOL

default-router 192.168.1.1

4

Renew Lease on Client

Verification Step

4.Renew Lease on Client:Verification Step.

On the client PC command prompt, force a lease renewal:

DOS

ipconfig /release

ipconfig /renew

Run ipconfig to verify the Default Gateway now displays 192.168.1.1.

3. Diagnosing "IP Works, Domain Name Fails"

Diagnostic Steps using nslookup / dig

- Confirm the Symptoms:

  - ping 8.8.8.8 →Success (Layer 3 IP Connectivity is functional).

  - ping google.com →Fails with Ping request could not find host google.com.

- Diagnose with nslookup (Windows / Cross-platform):

DOS

nslookup google.com

  - Observed Error: DNS request timed out. or Server Unreachable.

- Query a Specific Public DNS Server directly:

DOS

nslookup google.com 8.8.8.8

  - If this succeeds: Your client's configured local DNS server IP is wrong, offline, or blocking queries.

  - If this fails: Port 53 (UDP/TCP) is likely blocked outbound by a local firewall or network router.

- Diagnose with dig (Linux / macOS):

Bash

dig google.com

Look at the ;; ANSWER SECTION:. If it is missing or shows connection timed out; no servers could be reached, check the system server settings:

Bash

cat /etc/resolv.conf

Solution

- Client Machine Fix: Change your adapter's primary DNS server to a reachable local DNS server or public DNS (e.g., Primary: 8.8.8.8, Secondary: 1.1.1.1).

  - Windows CLI Fix:

DOS

netsh interface ip set dns name="Ethernet" static 8.8.8.8

- Flush DNS Cache: Clear stale/corrupted local mappings:

DOS

ipconfig /flushdns

- Verification: Run nslookup google.com. You should now receive an Non-authoritative answer showing valid IPv4/IPv6 addresses.

4. APIPA Address (169.254.x.x) Resolution Process

When a client shows an IP address in the range 169.254.0.1 – 169.254.255.254, it indicates that the client requested an IP via DHCP but received no response before timing out.

Root Causes

- Physical/Datalink Disconnect: Cable unplugged, bad Wi-Fi credentials, or misconfigured VLAN.

- DHCP Server Unreachable: DHCP service stopped, server down, or blocked by a firewall.

- Scope Exhaustion: The DHCP server has run out of available IP addresses to assign.

- DHCP Relay Failure: Routers are not forwarding DHCPDISCOVER broadcast messages across subnets (missing ip helper-address).

Step-by-Step Resolution Guide

1

Check Layer 1 & 2 Connectivity

Step 1

1.Check Layer 1 & 2 Connectivity:Step 1.

Verify link light activity on the network adapter/switch port. Ensure the client is assigned to the correct VLAN.

Inspect DHCP Server Status & Scope

Step 2

2.Inspect DHCP Server Status & Scope:Step 2.

Log into the DHCP server and verify the service status (active (running)). Open the scope pool address leases: if free addresses show 0, expand the subnet mask or reduce lease duration to reclaim stale IPs.

Verify DHCP Relay / Helper Address

Step 3

3.Verify DHCP Relay / Helper Address:Step 3.

If the client and DHCP server are on different subnets, verify the local router interface configuration includes the DHCP relay target:

Plaintext

interface GigabitEthernet0/0/1

ip helper-address 10.0.0.50

Release and Renew Lease

Step 4

4.Release and Renew Lease:Step 4.

On the affected client machine, release the APIPA address and request a new lease:

DOS

ipconfig /release

ipconfig /renew

Verification: Run ipconfig (or ip a on Linux) and confirm the adapter has received a valid IP address within your subnetwork range (e.g., 192.168.1.X or 10.X.X.X) rather than 169.254.X.X.

5. Common DNS and DHCP Issues Quick-Reference

| Technology | Issue | Symptoms | Possible Causes | Recommended Solution |
| --- | --- | --- | --- | --- |
| DNS | Incorrect Local DNS IP | Can access websites using direct IP addresses, but domain names show "Server Not Found". | Wrong DNS IP assigned by static configuration or DHCP server. | Change DNS settings to a working server (e.g., 8.8.8.8 or internal corporate DNS IP). |
| DNS | Corrupted DNS Cache | Browsers load outdated websites or throw connection errors after a server migration. | Local machine DNS cache holds stale IP mappings. | Flush DNS cache (ipconfig /flushdns on Windows or restart mDNSResponder on Mac). |
| DNS | Blocked Port 53 | nslookup queries time out completely for external domains. | Network firewall or ISP blocking UDP/TCP Port 53. | Allow outbound UDP/TCP Port 53 traffic on firewall rules, or switch to DNS over HTTPS (DoH). |
| DHCP | Scope Exhaustion | New devices receive 169.254.x.x (APIPA) addresses while existing devices work fine. | All available IP addresses in the DHCP pool are leased out. | Expand the IP address pool range, decrease lease time, or purge inactive leases. |
| DHCP | Rogue DHCP Server | Devices get unexpected IP ranges, wrong gateways, or lose connection to local servers. | Unmanaged router or unauthorized software running DHCP connected to switch. | Identify port using traceroute, disable rogue device port on switch, enable DHCP Snooping. |
| DHCP | Relay Agent Missing | Clients on subnets without a local DHCP server receive APIPA addresses. | Router isn't forwarding DHCP broadcast requests across network segments. | Configure ip helper-address <DHCP-Server-IP> on the router's client-facing interface. |