Day -6 : Build and Configure Small Office

1. Static IP Address Configuration & Connectivity Verification

Assigning static IPs ensures hosts maintain predictable network addresses without relying on standard DHCP leasing.

Setup Instructions

- Choose a Subnet: For a standard Class C private network, use network 192.168.1.0/24 (Subnet Mask: 255.255.255.0).

- Assign Host IPs:

  - PC-1: 192.168.1.10

  - PC-2: 192.168.1.11

  - PC-3: 192.168.1.12

  - PC-4: 192.168.1.13

  - PC-5: 192.168.1.14

- Configure Devices:

  - Windows/Packet Tracer PC: Go to Network Settings → IPv4 Properties → Select Use the following IP address → Enter IP, Subnet Mask (255.255.255.0), and Gateway (192.168.1.1 optional for intra-subnet).

  - Linux/macOS: Configure via Network Manager or CLI (sudo ip addr add 192.168.1.10/24 dev eth0).

1.Configure IP Addresses:Prerequisite.

Assign the IP addresses manually on all five devices within the 192.168.1.0/24 range.

2.Test Intra-Subnet Ping:Verification.

Open the command prompt/terminal on PC-1 (192.168.1.10) and run:

Bash

ping 192.168.1.11

ping 192.168.1.12

Verification: You should see Reply from 192.168.1.11: bytes=32 time<1ms TTL=128 with 0% packet loss.

2. Dynamic IP Assignment via DHCP Server

DHCP automates IP, Gateway, and DNS distribution to network clients.

Setup Instructions (Packet Tracer / Cisco IOS)

- Connect 3 client devices to a switch connected to a Cisco Router (or dedicated DHCP Server).

- Configure the DHCP pool on the Router CLI:

Plaintext

Router(config)# ip dhcp pool LAN_POOL

Router(dhcp-config)# network 192.168.10.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.10.1

Router(dhcp-config)# dns-server 192.168.10.2

Router(dhcp-config)# exit

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.9

- On each of the three client devices, open Network Settings and switch the IPv4 configuration from Static to DHCP.

Verification: Run ipconfig (Windows) or ifconfig / ip a (Linux/Mac) on each client. Confirm each device receives a unique IP in the range 192.168.10.10 to 192.168.10.254.

3. Local DNS Service & Custom Hostname Mapping

DNS maps human-readable domain names to numerical IP addresses.

Method A: Software DNS Server (dnsmasq / Cisco Packet Tracer)

- Add a Server device to the topology with static IP 192.168.1.2.

- Enable the DNS Service on the server.

- Add a new DNS Resource Record:

  - Name: musicplayer.local

  - Type: A Record

  - Address: 192.168.1.50 (IP of the target web server / media interface).

- Configure clients to use 192.168.1.2 as their Preferred DNS Server.

Method B: Quick Test via Local Hosts File

If testing locally without a DNS server, edit the operating system's hosts file on each device:

- Windows Path: C:\Windows\System32\drivers\etc\hosts

- Linux/macOS Path: /etc/hosts

Add the following entry at the bottom:

Plaintext

192.168.1.50    musicplayer.local

Verification: Open a web browser on any client device and navigate to [http://musicplayer.local](http://musicplayer.local). The target web page hosted at 192.168.1.50 should load successfully.

4. VLAN Segmentation & Inter-VLAN Isolation

VLANs broadcast domain boundaries at Layer 2 to logically segregate network traffic without needing separate physical switches.

Configuration Steps (Cisco Switch)

- Create VLANs on Switch:

Plaintext

Switch(config)# vlan 10

Switch(config-vlan)# name Workstations

Switch(config-vlan)# vlan 20

Switch(config-vlan)# name Smart_Devices

Switch(config-vlan)# exit

- Assign Switchports to VLANs:

  - PC-1 & PC-2 → VLAN 10 (Workstations)

  - PC-3 & PC-4 → VLAN 20 (Smart Devices)

Plaintext

! Assign Ports FastEthernet 0/1 - 0/2 to VLAN 10

Switch(config)# interface range fa0/1 - 2

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

! Assign Ports FastEthernet 0/3 - 0/4 to VLAN 20

Switch(config)# interface range fa0/3 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

1.Test Intra-VLAN Communication:VLAN 10 Internal.

From PC-1 (VLAN 10), ping PC-2 (VLAN 10).

2.Test Inter-VLAN Isolation:VLAN 10 to VLAN 20.

From PC-1 (VLAN 10), ping PC-3 (VLAN 20).

Verification:

- Intra-VLAN ping (PC-1 → PC-2): Success (0% loss).

- Inter-VLAN ping (PC-1 → PC-3): Timed Out / Failed (100% loss), proving broadcast and traffic isolation between VLANs.

5. NAT (Network Address Translation) Configuration for Internet Access

NAT translates internal private IP addresses (RFC 1918) into a public IP address before forwarding packets out to external networks.

Topology Overview

- LAN Side (Inside Interface): GigabitEthernet 0/0 → 192.168.1.1/24

- WAN/Internet Side (Outside Interface): GigabitEthernet 0/1 → 203.0.113.2/30

- Simulated External Server: 203.0.113.10 (Simulating www.wikipedia.org)

Interface Orientation Risk

Ensure inside (ip nat inside) and outside (ip nat outside) commands are assigned to the correct physical router interfaces. Inverting these commands will break translation and block traffic flow.

Cisco Router Configuration

Plaintext

! 1. Define Inside and Outside Interfaces

Router(config)# interface gigabitethernet 0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# ip nat inside

Router(config-if)# no shutdown

Router(config)# interface gigabitethernet 0/1

Router(config-if)# ip address 203.0.113.2 255.255.255.252

Router(config-if)# ip nat outside

Router(config-if)# no shutdown

Router(config-if)# exit

! 2. Define an ACL to identify local internal traffic

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

! 3. Enable PAT (Port Address Translation / NAT Overload)

Router(config)# ip nat inside source list 1 interface gigabitethernet 0/1 overload

! 4. Add Default Route to Internet

Router(config)# ip route 0.0.0.0 0.0.0.0 gigabitethernet 0/1

Verification:

- From a internal LAN device (192.168.1.10), ping external server 203.0.113.10 or open www.wikipedia.org in the browser.

- Check active NAT translations on the router CLI:

Plaintext

Router# show ip nat translations

Successful translation output example:

Pro Inside global Inside local Outside local Outside global

icmp 203.0.113.2:1024 192.168.1.10:1024 203.0.113.10:1024 203.0.113.10:1024