COMPUTER ARCHITECTURE & SYSTEMS LAB

# Day 3 Assignment: RAM & Storage Systems

TASK 1: MEMORY ARCHITECTURE COMPARISON

1. Generational RAM Standards Matrix (DDR3 vs. DDR4 vs. DDR5)

Synchronous Dynamic Random-Access Memory (SDRAM) has evolved rapidly to reduce operating voltages, maximize data bandwidth, and handle high core-count processor signaling paths.

Topic: Volatile vs. Non-Volatile Memory Topologies, Generational Memory Deltas, and Physical Media Interfaces

| Specification | DDR3 Memory | DDR4 Memory | DDR5 Memory |
| --- | --- | --- | --- |
| Data Rates / Speed | 800 MT/s to 2133 MT/s | 1600 MT/s to 3200 MT/s | 4800 MT/s to 8400+ MT/s |
| Operating Voltage | 1.5V Standard (1.35V Low Voltage) | 1.2V Base Architecture | 1.1V Base (On-DIMM PMIC regulation) |
| Channel Layout | Single 64-bit wide sub-channel | Single 64-bit wide sub-channel | Dual independent 32-bit sub-channels |
| Typical Use Cases | Legacy systems (e.g., Intel Core 3rd/4th Gen), older office PCs, thin clients. | Standard modern desktops, gaming systems, enterprise workstations (DDR4 server nodes). | Next-gen computing, heavy data compilation, real-time AI modeling, 4K/ 8K video production. |

TASK 2: PRACTICAL LAB COMPONENT

# RAM Extraction and Reinstallation Documentation

### SAFETY PROTOCOL: Disconnect all AC power cords, extract the internal battery if servicing a laptop, and ground yourself against electrostatic discharge (ESD) prior to interacting with the components.

[Student Lab Documentation — Installed RAM Module Verification]

Insert a clear photo showing the memory module seated inside your device's DIMM/SO-DIMM slot.

Ensure the alignment notch position or the module labeling is legible.

# 3. Mechanical vs. Solid-State Storage Analysis

Storage hierarchies balance raw capacity costs against input/output operations per second (IOPS) and data bus latencies.

| Storage Interface | Key Architectural Deltas & Speed Profiles | Optimized Target Application |
| --- | --- | --- |
|  |  | Mass Cold Backup Data |
|  | Mechanical spinning magnetic platters read by | Storage |
| Hard Disk Drive | an actuator arm. Prone to mechanical shock. | Archiving vast raw |
| (HDD) | Maximum operational sequential throughput tops | multimedia media files or |
|  | out near ~150–250 MB/s. High seek times. | system backups where |
|  |  | latency is secondary. |
|  | Flash NAND memory architecture bound to the | Operating System |
| SATA Solid-State Drive (SSD) | legacy SATA legacy controller protocol. Caps out structurally at an interface ceiling of ~550–600 MB/s due to link limitations. Zero moving | Upgrades Revitalizing legacy hardware systems lacking native M.2 |
|  | components. | motherboard slots. |
|  |  | Modern Gaming & IDE |
|  | Flash NAND running on the Non-Volatile | Workloads |
| NVMe Solid-State Drive (PCIe) | Memory Express standard communicating over direct high-bandwidth PCIe pathways. Achieves sequential reads from 3,500 MB/s to 14,000+ | Instant loading in asset-heavy software engine environments (e.g., modern |
|  | MB/s. Ultra-low queue latency. | games or local database |
|  |  | compilation). |

TASK 4: STORAGE INFRASTRUCTURE MAINTENANCE

Non-Volatile  Drive Interface Removal Log

### System Reassembly Post-Mortem Verification:

Upon seating the memory sticks completely into the retention mechanisms and clamping down the latch locks, the machine successfully passed the Power-On Self-Test (POST). System memory registration parameters confirm full configuration density at native speeds in the OS Task Manager.

TASK 3: NON-VOLATILE STORAGE TYPOLOGIES

Verification Step: Safely pull out your system's primary storage block or identify the physical form factor of your equipment via an approved manufacturer deployment framework documentation process.

[Student Lab Documentation — Storage Unit Connector Interface]

Insert a photographic trace or video process link demonstrating the drive pin connector footprint.

Identify if it uses a 2.5-inch SATA bracket setup, a 2280 M.2 layout keyway, or an NVMe slot channel.

:

Device Storage Layout Structure Classification: 	 (e.g., M.2 M-Key NVMe PCIe Gen 4 / 2.5-inch SATA Data and Power Interface Header). Internal BIOS utility registers the drive index correctly following reassembly.